MSc Dissertation — COMP5200M

# Proposed Experimental Setup

Review Before Training

Purpose: Please review the proposed instance configurations before I proceed with full-scale training.

# 1. Instance Configurations

Eight configs covering single-machine and multi-machine problem sizes. Naming: n{jobs}_m{machines}.

| Config | Jobs | Machines | μ (hrs) |
| --- | --- | --- | --- |
| n5_m1 | 5 | 1 | 33.6 |
| n10_m1 | 10 | 1 | 16.8 |
| n20_m1 | 20 | 1 | 8.4 |
| n50_m1 | 50 | 1 | 3.36 |
| n100_m1 | 100 | 1 | 1.68 |
| n20_m3 | 20 | 3 | 25.2 |
| n50_m5 | 50 | 5 | 16.8 |
| n500_m10 | 500 | 10 | 3.36 |

Tier 1 — Small (n ≤ 20): Baseline comparisons. GA adequate at these sizes.

Tier 2 — Medium (n = 50–100): Core evaluation. Search space large enough for adaptive mutation to matter.

Tier 3 — Large (n = 500): Scalability test. Industrial-scale instance.

# 2. Scalability Considerations

Question: Should I add larger configs (n ≥ 1000), or is n500_m10 sufficient?

Coverage: The current set spans 5 to 500 jobs (three orders of magnitude). This is sufficient to observe scaling trends.

Diminishing returns: Beyond n=500, runtime increases substantially without qualitatively new insight. The permutation space is already 500! ≈ 10⁻¹ʸ⁵.

Scope: Configs beyond n500 can be noted as future work in the thesis.

# 3. Instance Generation Model

Instances are synthetic, grounded in 24/7 multi-shift factory operation (168 hrs/week).

Processing time: pᵢ ~ Uniform(0.3μ, 1.7μ) where μ = (m × 168)/n. Total work fills one week.

Setup cost: Asymmetric. Dark→light: (diff) × 10. Light→dark: |diff| × 3. Diagonal = 0.

Setup time: Normalised cost × μ/8. Average cleaning ≈ 1/8 of dye cycle.

Due dates: Proportional within week, SPT order + jitter.

# 4. Next Steps

After your feedback:

1. Run baselines (SPT + NN-Greedy), 50 seeds per config.

2. Train PPO (100k timesteps).

3. Run standalone GA, 50 seeds per config.

4. Run hybrid (GA + PPO), 50 seeds per config.

5. Sensitivity analysis (α ∈ {0.3, 0.5, 0.7}).

6. Statistical evaluation and visualisation.

Estimated runtime: ~3–6 hours per full set on cloud VM.